广西地方标准《烟叶表面高级脂肪烃的测定 气相色谱质谱法》（征求意见稿）编制说明

一、任务来源

广西地方标准《烟叶表面高级脂肪烃的测定 气相色谱质谱法》由广西烟叶标准化技术委员会提出，广西壮族自治区质量技术监督局批准立项，并列入《广西壮族自治区质量技术监督局关于下达2017年第六批广西地方标准制定（修订）项目计划的通知》（桂质监函〔2017〕242号）文件中（项目序号：2017-0640），由广西中烟工业有限责任公司起草，计划2018年完成。

二、编制背景及意义

烟叶表面高级脂肪烃是构成植物表皮蜡质的主要成分，烟叶上每100cm2含有5～10mg烷烃，其含量随烟叶的叶龄增加，且叶片位置较高的烟叶的烷烃更多。烟叶表面高级脂肪烃对烟叶颜色、光泽、有份、叶片结构、厚度均具有一定的影响作用。对烤烟香气和吃味有重要影响。

高级脂肪烃为植物进行自我防护提供重要保障，可以在烟叶适应干旱环境以及各种非生物和生物逆境过程中发挥重要作用，例如：限制非气孔性水分散失、抵抗紫外辐射、抵御病虫害以及防止干旱危害植物内部组织等。

同时，高级脂肪烃是通过长链脂肪酸脱羟基作用合成，对气候、环境变化敏感，经研究表明，高级脂肪烃的烷烃分子组成与环境条件密切相关，环境条件及植物生理作用的差别可能导致植物高级脂肪烃种内分布变化较大。

经研究结果表明，高级脂肪烃种类与烟叶保润性能呈显著正相关，开展烟叶中高级脂肪烃的分析研究，对提高烟叶品质、探索烟草物理保润机理，研究不同地域烟叶品质的变化特点具有重要的作用。

目前，国内外对烟草中高级脂肪烃的研究相对较少，且前处理、检测方法各异，不同项目组分析数据差异较大，很难进行统一的比较和分析。因此，制定《烟叶表面高级脂肪烃的测定 气相色谱质谱法》地方标准，对于统一烟叶表面高级脂肪烃的检测方法，研究不同烟叶表面高级脂肪烃的含量分布情况具有深远的意义。

三、标准编制过程

广西地方标准《烟叶表面高级脂肪烃的测定 气相色谱质谱法》任务下达后，广西中烟工业有限责任公司成立了编制工作小组，制订了《〈烟叶表面高级脂肪烃的测定 气相色谱质谱法〉编写方案与实施计划表》，调研国内外相关资料完成文献调研，经过对标准开展方法研究实验、方法优化实验，方法验证、共同实验等步骤，完成该标准讨论稿，选取5家实验室开展共同实验，对标准讨论稿进行比对试验，验证评价本方法的可行性，形成征求意见稿。

四、标准编制原则和依据

原则：遵循科学性、适用性、先进性的原则，力求做到科学规范、指标准确、操作易行。

依据：按照 GB/T 1.1－2009《标准化工作导则第1部分：标准的结构和编写规则》所规定的内容和格式编制。

五、标准的主要内容及依据和来源

（一）高级脂肪烃性质

植物中烷烃主要分布于叶片或者果实的表面，会在其表面形成一层蜡质类的屏障，蜡质层不仅为维持叶片或者果实的内部环境提供保障，减弱紫外线对植物的损害，而且可以减少植物中自由水的损失。

烟叶表面蜡质成分主要以为烷烃为主，烟叶上每100cm2含有5～10mg烷烃化合物，其含量随烟叶的叶龄主检增加，叶位较高的烟叶比较低的烟叶含有更多的烷烃，具有27～33个碳原子的成分占烷烃总量的90%以上。可接受的烤烟、白肋烟、香料烟样品中总石蜡族烃类的含量无明显变化。

对云烟87初烤烟叶B2F 中C12-C35烷烃进行了检测（结果见表1），发现上述碳数范围内的烷烃总量为748.66μg/g，不同烷烃含量差别较大，二十七碳至三十三碳合计有717.43μg/g，占测定烷烃总量的95.83%，因此下一步分析中将选择C27-C33烷烃进行分析。

表1  云烟87B2F初烤烟叶中烷烃的种类与含量

| 名称 | 含量（μg/g） | 名称 | 含量（μg/g） |
| --- | --- | --- | --- |
| 正十二烷 | 0.6562 | 正二十四烷 | 0.425 |
| 正十三烷 | 0.3376 | 正二十五烷 | 8.1198 |
| 正十四烷 | 0.8122 | 正二十六烷 | 4.2776 |
| 正十五烷 | 0.3255 | 正二十七烷 | 74.453 |
| 正十六烷 | 0.6161 | 正二十八烷 | 10.1777 |
| 正十七烷 | 0.5031 | 正二十九烷 | 91.5761 |
| 正十八烷 | 0.3548 | 正三十烷 | 34.3207 |
| 正十九烷 | 0.5514 | 正三十一烷 | 308.0789 |
| 正二十烷 | 0.6612 | 正三十二烷 | 58.4838 |
| 正二十一烷 | 0.3431 | 正三十三烷 | 140.3365 |
| 正二十二烷 | 0.3965 | 正三十四烷 | 5.7805 |
| 正二十三烷 | 0.3439 | 正三十五烷 | 6.7297 |

（二）检测方法的线性范围和检出限、定量限、精密度

配制高级脂肪烃标准溶液系列5.0、10.0、20.0、100.0、500.0µg/mL，体系内内标浓度为50µg/mL，按照本标准的检测方法进行实验。横坐标为各高级脂肪烃峰面积与内标物峰面积比值，纵坐标为各高级脂肪烃的浓度和内标物浓度的比值，建立标准工作曲线。以3倍信噪比计算检出限，10倍信噪比计算方法定量限。精密度为同一样品重复测定6次的RSD%。

表2  标准曲线、检出限和定量限、精密度

| 名称 | 保留时间/min | 标准曲线 | R2 | 检出限/ | 定量限/ |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | (μg/g) | (μg/g) | RSD% |
| 正二十七烷 | 56.98 | y=38.7090x+7.7744 | 0.9997 | 0.05 | 0.17 | 1.73 |
| 正二十八烷 | 60.18 | y=37.7090x+1.0690 | 0.999 | 0.04 | 0.13 | 2.67 |
| 正二十九烷 | 62.94 | y=39.3280x+5.8711 | 0.9999 | 0.04 | 0.15 | 2.06 |
| 正三十烷 | 65.21 | y=38.1050x+11.553 | 0.9992 | 0.04 | 0.15 | 1.85 |
| 正三十一烷 | 67.00 | y=41.7570x+2.3240 | 1 | 0.04 | 0.15 | 1.53 |
| 正三十二烷 | 68.66 | y=43.1250x+1.6462 | 1 | 0.05 | 0.15 | 2.39 |
| 正三十三烷 | 70.15 | y=41.3470x+0.2892 | 1 | 0.04 | 0.13 | 2.8 |

（三）检测方法的回收率

为了验证本标准检测方法的准确性和可靠性，我们进行了高中低三个水平的加标回收实验，结果见表1。

表3  回收率、重复性

| 名称 | 回收率% |  |  |
| --- | --- | --- | --- |
|  | 低 | 中 | 高 |
| 正二十七烷 | 73.67 | 81.33 | 80.08 |
| 正二十八烷 | 81.35 | 91.7 | 90.6 |
| 正二十九烷 | 72.2 | 80.31 | 78.82 |
| 正三十烷 | 78.45 | 74.09 | 71.74 |
| 正三十一烷 | 70.48 | 78.71 | 75.51 |
| 正三十二烷 | 71.55 | 75.31 | 73.24 |
| 正三十三烷 | 72.49 | 77.24 | 106.69 |

从表3中可见，不同添加量样品的回收率在70.0%～107.0%之间。样品检测重复性RSD%在5%以内。

五、结论

本标准采用气相色谱-质谱法（GC-MS）检测烟叶表面高级脂肪烃，我们通过大量的实验对样品处理、色谱分离条件和质谱定性、定量参数进行了筛选和优化，并采用加标回收实验和重复平行实验对检测方法进行了验证，结果表明该标准的检测方法能够满足烟叶表面高级脂肪烃的测定。

六、参考文献

[1] Heemann V，Brilmmer U，Paulsen C，Seehofer F．Composition of the leaf surface gum of some Nicotiana species and Nicotiana tabacum cultivars[J]. Phytochemistry, 1983, 22(1): 133—135．

[2] Grice K, Lu H, Zhou Y, Stuart-Williams H, Farquhar G.D. Biosynthetic and environmental effects on the stable carbon isotopic compositions of anteiso-(3-methyl) and iso-(2-methyl) alkanes in tobacco leaves[J]. Phytochemistry. 2008, 69: 2807–2814.

[3]Severson R F, Arrendale R F, Chortyk O T, et a1．Quantitation of the major euticular components from green leaf of different tobacco typos[J]．J Agric Food Chem, 1984, 32: 566-570.

[4]Court W A. Factors affecting the concentration of the duvaterienediols of flue-cured tobacco[J]. Tob Sci, 1982, 26: 40-43.

[5]Kandral L，Wanger G J. Study of the site and mode of biosynthesis of tobacco trichome exeudate components[J]. Arch Biochem Biophys, 1988, 265: 425-432.

[6] Lynda Webster, Pamela Simpson, Aileen M. Shanks, Colin F. Moffat. The authentication of olive oil on the basis of hydrocarbon concentration and composition [J]. Analyst, 2000, 125: 97-104.

[7] O. Koprivnjak, S. Moret, T. Populin, C. Lagazio, L.S. Conte. Variety differentiation of virgin  olive oil based on n-alkane profile[J]. Food Chemistry, 2005, 90: 603-608.

[8] Wahid Herchi, Saoussem Harrabi, Sophie Rochut, et al. Characterization and Quantification of the Aliphatic Hydrocarbon Fraction during Linseed Development [J]. Food Chem. 2009, 57: 5832–5836.

[9] D. N. Gabov, V. A. Beznosikov, et al. Saturated Hydrocarbons in the Background and Contaminated Soils of the Cisurals[J]. Soil Chemsitry, 2010, 10: 1190–1196.

[10] C. G. Blanco, J. G. Prado, Preliminary results of extraction experiments in an oil shale. Org. Geochem. 1992, 18: 313-316.

[11]阳会兵, 周清明, 杨虹琦, 等. 薄层色谱扫描法测定烟叶表面蔗糖酯含量[J]. 湖南农业大学学报（自然科学版）, 2007, 33(2): 177-179.

[12]朱显灵, 陈伟, 徐增涵, 等. 贵州不同生态产区有机烟叶烤后烟叶表面提取物的含量[J]. 贵州农业科学, 2011, 39(1): 34-37.

[13]Einolf W N, Chan W G. Estimation of sucrose estersin tobacco by direct chemical ionization mass spoctrometry [J]. J Agric Food Chem, 1984, 32: 785-789.

[14]何琴, 刘菊, 徐忠辉等. 烟草中异构烷烃和反异构烷烃的成因及其光合作用C同位素分馏效应[J]. 地球化学, 2009, 38(3): 282-288.

[15] Xianyu Huang, Philip A. Meyers, et al. Significance of long chain iso and anteiso monomethyl alkanes in the Lamiaceae (mint family)[J]. Organic Geochemistry 42 , 2011: 156–165.

[16]屈慧, 杨志学. 马蹄莲中烷烃的分离与结构鉴定[J]. 中国药师. 2009, 12(10): 1361-1362.

[17]Khorassani M A, Taylor L T, Nazem N, et a1.Isolation of tetral-acyl sucrose esters from Turkish tobacco using suporcritical fluid C02 and comparison with conventional solvent extraction[J]. J Agric Food Chem, 2005, 53(6): 1866-1872.

[18]Danehower D A, Lawson D R. A thin layer densitometry technique for the rapid quanititation of Nicotiana tabaccum leaf surface compounds[c]. Tob Res Conf, 1988.

[19]Seveetson R. F. Quantitative gas chromatographic of aliphatic hydrocarbons, rerpenes, fatty alcohol, fatty acids and sterols in tobacco[J]. Journal of Chromatography, 1978.

[20] Nip M, Tegelaar E W, Analysis of modern and fossil plant cuticles by Curie point Py-GC and Curie point Py-GC-MS: Recognition of a new, highly aliphatic and resistant biopolymer[J]. organ geochemistry,1986.

[21]李宝才, 董玉莲等, 秋茄和榕树叶片中正构烷烃分布和单体化合物13C值及其光合作用[J]. 热带海洋学报, 2003.

[22] Xin L, Mingyue Z, Hongwei.et al. Characterization of cigarette smoke condensates by comprehensive two-dimensional gas chromatography/time-of-flight mass spectrometry (GC×GC/TOFMS) Part 2: Basic fraction[J], Journal of Separation Science, 2004, 27: 101-109.

[23]Iliasg.Kavouras, Nikolaos Stratigakis, et al. Iso- and Anteiso-Alkanes Specific tracers of environmental tobacco smoker in indoor and outdoor partical-size distributed urban aerosols[J]. Environ. Sci. Techno l. 1998, 32: 1369-1377.

标准编写小组

2018年11月6日